AIM: Design Butterworth and Chebyshev filter using bilinear transformation method.

Theory: The bilinear transformation method is commonly used to design analog filters and then convert them into digital filters. This method maps the analog frequency response to the digital frequency response using a bilinear transformation.

The Butterworth and Chebyshev filters are two commonly used filter types. The Butterworth filter has a maximally flat frequency response in the passband, while the Chebyshev filter allows for a sharper transition between the passband and the stopband at the expense of ripples in either the passband or stopband.

Program:

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import butter, bilinear, freqz, cheby1

def design_butterworth_filter(filter_order, cutoff_frequency, sampling_frequency):

# Design the analog Butterworth filter

analog_b, analog_a = butter(filter_order, cutoff_frequency, analog=True, btype=&#39;low&#39;)

# Perform the bilinear transformation

digital_b, digital_a = bilinear(analog_b, analog_a, sampling_frequency)

return digital_b, digital_a

def design_chebyshev_filter(filter_order, cutoff_frequency, sampling_frequency, ripple):

# Design the analog Chebyshev filter

analog_b, analog_a = cheby1(filter_order, ripple, cutoff_frequency, analog=True, btype=&#39;low&#39;)

# Perform the bilinear transformation

digital_b, digital_a = bilinear(analog_b, analog_a, sampling_frequency)

return digital_b, digital_a

def plot_filter_response(digital_b, digital_a, sampling_frequency):

# Compute the frequency response of the filter

frequency, magnitude_response = freqz(digital_b, digital_a, fs=sampling_frequency)

# Plot the magnitude response

plt.figure(figsize=(10, 6))

plt.plot(frequency, np.abs(magnitude_response))

plt.title(&#39;Filter Magnitude Response&#39;)

plt.xlabel(&#39;Frequency (Hz)&#39;)

plt.ylabel(&#39;Magnitude&#39;)

plt.grid(True)

plt.show()

# Compute the impulse response of the filter

_, impulse_response = freqz(digital_b, digital_a, fs=sampling_frequency, worN=4096)

# Plot the impulse response

plt.figure(figsize=(10, 6))

plt.plot(impulse_response)

Marwadi University

Faculty of Engineering and Technology

Department of Information and Communication Technology

plt.title(&#39;Filter Impulse Response&#39;)

plt.xlabel(&#39;Samples&#39;)

plt.ylabel(&#39;Amplitude&#39;)

plt.grid(True)

plt.show()

# Specify the desired filter specifications

filter_order = 4 # Filter order

cutoff_frequency = 1000 # Cutoff frequency in Hz

sampling_frequency = 8000 # Sampling frequency in Hz

ripple = 0.5 # Ripple factor for Chebyshev filter

# Design the Butterworth filter

digital_b, digital_a = design_butterworth_filter(filter_order, cutoff_frequency, sampling_frequency)

# Plot the Butterworth filter&#39;s magnitude response and impulse response

plot_filter_response(digital_b, digital_a, sampling_frequency)

# Design the Chebyshev filter

digital_b, digital_a = design_chebyshev_filter(filter_order, cutoff_frequency, sampling_frequency,

ripple)

# Plot the Chebyshev filter&#39;s magnitude response and impulse response

plot_filter_response(digital_b, digital_a, sampling_frequency)

# Save the filter coefficients (optional)

filter_path = &#39;filter_coefficients.txt&#39;

np.savetxt(filter_path, np.vstack((digital_b, digital_a)), delimiter=&#39;,&#39;)

print(f&quot;Filter coefficients saved at: {filter_path}&quot;)

Code:-

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import butter, bilinear, freqz, cheby1

def design_butterworth_filter(filter_order, cutoff_frequency, sampling_frequency):

# Design the analog Butterworth filter

analog_b, analog_a = butter(filter_order, cutoff_frequency, analog=True, btype='low')

# Perform the bilinear transformation

digital_b, digital_a = bilinear(analog_b, analog_a, sampling_frequency)

return digital_b, digital_a

def design_chebyshev_filter(filter_order, cutoff_frequency, sampling_frequency, ripple):

# Design the analog Chebyshev filter

analog_b, analog_a = cheby1(filter_order, ripple, cutoff_frequency, analog=True, btype='low')

# Perform the bilinear transformation

digital_b, digital_a = bilinear(analog_b, analog_a, sampling_frequency)

return digital_b, digital_a

def plot_filter_response(digital_b, digital_a, sampling_frequency):

# Compute the frequency response of the filter

frequency, magnitude_response = freqz(digital_b, digital_a, fs=sampling_frequency)

# Plot the magnitude response

plt.figure(figsize=(10, 6))

plt.plot(frequency, np.abs(magnitude_response))

plt.title('Filter Magnitude Response')

plt.xlabel('Frequency (Hz)')

plt.ylabel('Magnitude')

plt.grid(True)

plt.show()

# Compute the impulse response of the filter

_, impulse_response = freqz(digital_b, digital_a, fs=sampling_frequency, worN=4096)

# Plot the impulse response

plt.figure(figsize=(10, 6))

plt.plot(impulse_response)

plt.title('Filter Impulse Response')

plt.xlabel('Samples')

plt.ylabel('Amplitude')

plt.grid(True)

plt.show()

# Specify the desired filter specifications

filter_order = 4              # Filter order

cutoff_frequency = 1000       # Cutoff frequency in Hz

sampling_frequency = 8000     # Sampling frequency in Hz

ripple = 0.5                  # Ripple factor for Chebyshev filter

# Design the Butterworth filter

digital_b, digital_a = design_butterworth_filter(filter_order, cutoff_frequency, sampling_frequency)

# Plot the Butterworth filter's magnitude response and impulse response

plot_filter_response(digital_b, digital_a, sampling_frequency)

# Design the Chebyshev filter

digital_b, digital_a = design_chebyshev_filter(filter_order, cutoff_frequency, sampling_frequency, ripple)

# Plot the Chebyshev filter's magnitude response and impulse response

plot_filter_response(digital_b, digital_a, sampling_frequency)

# Save the filter coefficients (optional)

filter_path = 'filter_coefficients.txt'

np.savetxt(filter_path, np.vstack((digital_b, digital_a)), delimiter=',')

print(f"Filter coefficients saved at: {filter_path}")

Output:

Conclusion:

In this experiment, Butterworth and Chebyshev digital filters were successfully designed from analog prototypes using the bilinear transformation method, effectively mapping the specified cutoff and sampling frequencies from the analog to the digital domain without aliasing. The generated plots validated the theoretical properties of both filter types: the Butterworth filter exhibited a maximally flat passband ideal for smooth, distortion-free signal processing, while the Chebyshev filter achieved a sharper transition between the passband and stopband by introducing a controlled ripple factor. Finally, the successful computation of their impulse responses and the export of the digital filter coefficients to a text file demonstrated that these filters are fully functional and ready for practical implementation in digital signal processing systems.

GITHUB LINK:-

https://github.com/SlockAhuja/DSIP.git